HA NOI UNIVERSITY OF SCIENCE AND TECHNOLOGY SCHOOL OF ELECTRICAL AND ELECTRONIC ENGINEERING PROJECT DIGITAL DESIGN II TOPIC: NEURON GRID BASED ON SPIKING NEURAL NETWORK Instructor: Assoc. Prof. Dr. NGUYEN DUC MINH Students: NGUYEN LE TRUNG 20186076 VU HOANG LONG 20182926 PHAM HUY HOANG 20182544 Ha Noi, 02-2022

PREFACE

This project report is submitted in fulfillment of the requirements for the course Digital Design II of  School of Electrical Engineering. We are required to make a project on “Neuron grid based on spiking neural network”. The basic objective behind carrying out this project requirement is to understand and apply SNN theory to practical design.

In this report, we included various concepts, measurement step and simulation results on Modelsim and Vivado.

Through this progress of doing this project, we come to enhance the teamwork and problem-solving skill.

We are grateful to outstanding course instructor Nguyen Duc Minh. He was instrumental in preparing our class essential guidance and references to be capable of completing this assignment and has been helpful throughout. It's an honor to have completed this course under his direction, which aided in bringing out the best of our efforts in order to achieve our objective.

TABLE OF CONTENTS

ABBREVIATIONS	1

FIGURES AND DIAGRAMS	2

TABLES	3

ABSTRACT	4

CHAPTER 1.	INTRODUCTION	5

1.1 Overview of Spiking Neuron Network	5

1.2 Overview of Reconfigurable Architecture for Neuromorphic Computing	6

CHAPTER 2.	SPECIFICATIONS	8

2.1 TOP module	10

2.2 Interface Signals	10

2.3 Function descriptions	11

2.4 Algorithm and ASMD	12

CHAPTER 3.	IMPLEMENTATION	16

3.1 Software	16

3.2 Hardware	16

3.2.1 ASMD code	17

3.2.2 Testbench	20

CHAPTER 4.	RESULT	23

CONCLUSION	24

REFERENCES	25

# ABBREVIATIONS

| SNN RANC FSM ASM ASMD | Spiking neural network Reconfigurable Architecture for Neuromorphic Computing Finite state machine Algorithmic state machine Algorithmic state machine with datapath |
| --- | --- |

# FIGURES AND DIAGRAMS

Figure 1 A natural Neuron	5

Figure 2 Potential of a Neuron through time	6

Figure 3 High level architectural overview of RANC components	6

Figure 4 Our Neuron Grid	8

Figure 5 Calculation process in Neuron Grid	8

Figure 6 TOP module of Neuron Grid	10

Figure 7 Timing diagram of Neuron Grid	11

Figure 8 Controlling and feedback signals	12

Figure 9 Flow chart of Neuron Grid (part 1)	12

Figure 10 Flow chart of Neuron Grid (part 2)	13

Figure 11 ASM diagram of Neuron Grid	14

Figure 12 TOP module of Neuron Block	15

Figure 13 Architecture of Neuron Block	15

# TABLES

Table 1 Synthesis results table	23

# ABSTRACT

In this project, a Neuron Grid is proposed using ASMD. The Neuron Grid will be designed and simulated on the ModelSim and synthesized on Vivado. This project introduces the involved theory and  demonstrates the process of designing a Neuron Grid from making the flowchart, ASM, implementing on C++, Verilog  and verifying the design. By which is divided in to 6 chapters explain detailly the design process :

CHAPTER 1. INTRODUCTION

An Introduction SNN and RANC

CHAPTER 2. SPECIFICATIONS

Neuron Grid information through specifications

CHAPTER 3. IMPLEMENTATION

Neuron Grid implementation on C++ and Verilog

CHAPTER 4. RESULTS

Results after synthesizing

# INTRODUCTION

## Overview of Spiking Neuron Network

Spiking neural network (SNN) is an artificial neural network that closely mimic natural neural network in the human brain which includes many Neurons connected to each other.

Figure 1 A natural Neuron

Some important parts of a natural Neuron:

- Dendrite: a branch of a Neuron.

- Axon: a cable where electrical impulses from a Neuron travel to other neurons.

- Synapse: a point of contact between an Axon and a Dendrite.

Neurons in SNN transmit information only when a membrane potential – an intrinsic quality of the neuron related to its membrane electrical charge – reaches a specific value, called the threshold. When the membrane potential reaches the threshold, the neuron fires, and generates a signal (called Spike) that travels to other Neurons which, in turn, increase or decrease their potentials in response to this signal:

Figure 2 Potential of a Neuron through time

## Overview of Reconfigurable Architecture for Neuromorphic Computing

Reconfigurable Architecture for Neuromorphic Computing (RANC) is an open source software ad hardware ecosystem that seeks to make neuromorphic architectures widely accessible to researchers and application developers through a cohesive programming and testing environment.

RANC uses a multicore architecture with each Core mimicking a part of natural neural network based on SNN. Spikes generated from Neurons in each Core are transmitted to other Core:

Figure 3 High level architectural overview of RANC components

- Five main parts of a Core in RANC:

- Neuron Block: primary computational component for the architecture.

- Core controller: a finite state machine (FSM) operating with a datapath (neuron block).

- Core SRAM: main memory of the architecture.

- Packet Router: component that is responsible for inter-core communication.

- Scheduler: final stop for a spike packet that is traversing the cores through the routing network.

# SPECIFICATIONS

Based on RANC, we redesign its Neuron Block and Core Controller and combine them into an architecture called Neuron Grid by using Algorithmic State Machine (ASMD).

Figure 4 Our Neuron Grid

We represent an architecture from a matrix N(a) x N(n) (with N(a) = 256 Axons and N(n) = 256 Neurons) in Figure 4.

Figure 5 Calculation process in Neuron Grid

Figure 5 shows that output is generated every time a Neuron is processed with all 256 Axons. Each Neuron is processed successively. Potential is cumulated when there is a Spike and a Synaptic connection.

Equation of integrated potential in a Neuron:

μ=vit-1+jspikej×wjτj+li, where j∈Ω

sit= 1           if   vi≥vi+0             otherwise

vit=ri+                      if μ≥vi+ri-                      if μ<vi-μ                       otherwise

where:  Ω	 is the set of all axons that connect to the current neuron

spikej	 indicates whether or not there is an incoming spike at j-th axon

μ	 is the integrated potential of the current neuron

vi (t) 	 is the potential of neuron i at time t

τj 	 is the neuron instruction corresponding to j-th axon

wjτj 	 is the τj-th kind of weight of axon j

li 	 is the leak potential of i-th neuron

si 	 indicates whether or not there is an outgoing spike at i-th neuron

vi+ 	 is the positive threshold

vi- 	 is the negative threshold

ri+ 	 is the positive reset potential

ri- 	 is the negative reset potential

## TOP module

Figure 6 TOP module of Neuron Grid

## Interface Signals

| Signal name | Width | I/O | Description |
| --- | --- | --- | --- |
| clk | 1 | Input | DTI Clock Signal |
| reset_n | 1 | Input | DTI Asynchronous Reset, active LOW |
| tick | 1 | Input | start working, active HIGH |
| neuron_instruction | 2 | Input | refers to the weight types of each neuron |
| neuron_parameter | 368 | Input | parameters of a neuron stored in RAM |
| axon_spikes | 256 | Input | spikes information of the entire network. Bit “1” indicates that there is a spike. |
| axon_num | 8 | Output | address where to take neuron_instruction from storage |
| neuron_num | 8 | Output | address where to take neuron_parameter from RAM |
| potential_out | 9 | Output | neuron potential after processed |
| spike_out | 1 | Output | indicate the moment that there is a spike |
| update_potential | 1 | Output | indicate the RAM to update potential_out into neuron_parameter |
| done | 1 | Output | indicate that the process finishes |
| sheduler_set | 1 | Output | request axon_spikes from the storage |
| scheduler_clr | 1 | Output | clear the information after taken |
| error | 1 | Output | indicate there is error |

## Function descriptions

368-bit neuron_parameters includes 256 bits synapse connections, 9 bits current potential, 9 bits reset potential, 36 bits weight (9 x 4), 9 bits leak value, 9 bits positive threshold, 9 bits negative threshold, 1 bit reset mode, 9 bits dx value, 9 bits dy value, 8 bits axon destination, 4 bits deliver tick offset. Not all the parameters are used in our Neuron Grid because they are based on RANC so some are for other blocks.

Asynchronous reset, active Low.

When tick is High, the network starts working. At the first positive edge of clock after tick is High, scheduler_set is High to request axon_spikes from the storage.

At the next negative edge of clock, neuron_num and axon_num is initialized and both are triggered by negative edge of clock. axon_num increases after 1 cycle of clock and neuron_num increases when axon_num is 255.

At the first positive edge of clock when axon_num is 255, spike_out is triggered. Potential_out is updated at the positive edge of clock when axon_num changes. Update_potential is High at positive edge of clock when axon_num reaches 255.

Scheduler_clr and done is High at the first positive edge of clock after scheduler_clr is High. Error is High at the positive edge of clock when tick is High during the proccess.

Figure 7 Timing diagram of Neuron Grid

## Algorithm and ASMD

Figure 8 Controlling and feedback signals

Figure 9 Flow chart of Neuron Grid (part 1)

Figure 10 Flow chart of Neuron Grid (part 2)

The flowchart is split into states to create ASM

IDLE: Initialize counting variable (num_spike_out) and finishing variable (done).

GET_DATA: Initialize index variable of neuron (i = 0) and assign integrated_potential = curent_potential

INITIAL: Initialize index variable of axon (j = 0). It is not suitable to combine this state and GET_DATA because with each neuron, all 256 axons are processed (j from 0 to 255).

SPIKE_IN: Perform the calculation for a neuron. The calculation is repeated for 256 neurons and the condition of spike_out and reset_mode is considered, so this process is in a state. The condition of spike_out and reset_mode can be considered in another state but it is not necessary because the calculation is not complex and does not affect the clock period significantly.

UPDATE: finish the process of saving new potential to RAM before the program is done.

END: finish the program, done =1 announces that the tick is done.

Figure 11 ASM diagram of Neuron Grid

In datapath of Neuron Grid, we have a sub module called Neuron Block to calculate potential and generate spikes with TOP module in Figure 12 and architecture in Figure 13.

Figure 12 TOP module of Neuron Block

Figure 13 Architecture of Neuron Block

# IMPLEMENTATION

## Software

To implement Neuron Grid, we initially apply the algorithm on C++. This software program can generate neuron_parameters and axon_spikes randomly and store in text files. However neuron_instruction used is from existing files from RANC. Details are mentioned in Guide to run experiment file.

To verify our C++ software, we compare its output with RANC hardware. We combine Core Controller and Neuron Block of RANC and write a test bench to compare its results with our Neuron Grid. The test bench read output of software program stored in text files.

The C++ software can create 3 types of file: text file containing axon spikes information in 1 tick, text file containing axon spikes information in many ticks and text file containing all the neuron parameters. They can be created by running 3 generating functions in main().

int main() { ///////////Generate//////////// generateCSRAM(); generateManyTickAxonSpikes(16); //16 tick generateAxonSpikes(); //1 tick //////////Simulation////////// NeuronGrid(); //1 tick ManyTick(16); //16 tick return 0; }

There are also 2 functions to simulate in 1 tick and many ticks in main().

We have functions to receive data from input text files and use the flowchart in Figure 9 to implement the calculation functions.

## Hardware

To implement Neuron Grid on Verilog, we use ASMD method. To verify Neuron Grid, we compare the results with results generated by software in text files.

### ASMD code

We map 1:1 ASMD to Verilog code.

In controller, conditions to change the states are in ASM’s condition block. Outputs of controller are shown in ASM’s process blocks. Always and sensitivity list are also used.  These rules are described in the following Verilog code in controller:

always @(current_state, tick, done_neuron, spike_in, done_axon) begin initial_axon_num = 0; initial_neuron_num = 0; scheduler_set = 0; scheduler_clr = 0; new_neuron = 0; process_spike = 0; inc_axon_num = 0; update_potential = 0; done = 0; inc_neuron_num = 0; spike_out = 0; ////// case(current_state) IDLE: begin next_state = tick ? GET_DATA : IDLE; end GET_DATA: begin initial_neuron_num = 1; scheduler_set = 1; next_state = INITIAL; new_neuron = 1; end INITIAL: begin initial_axon_num = 1; process_spike = 1; next_state = SPIKE_IN; end SPIKE_IN: begin process_spike = 1; if(done_axon) next_state = UPDATE; else begin inc_axon_num = 1; next_state = SPIKE_IN; end end UPDATE: begin update_potential = 1; spike_out = spike_in; next_state = END; end END: begin if(done_neuron) begin scheduler_clr = 1; done = 1; next_state = IDLE; end else begin new_neuron = 1; inc_neuron_num = 1; next_state = INITIAL; end end default: next_state = IDLE; endcase end

In controller, error happens when tick is High in all states except for IDLE which is shown in the following code:

always @(posedge clk, negedge reset_n) begin if(~reset_n) begin current_state <= IDLE; error <= 0; end else begin current_state <= next_state; if(~error && current_state != IDLE && tick) error <= ~error; else error <= error; end end

Inputs of datapath are shown in ASM’s process blocks. Sub-block neuron block is used when there are both synaptic connection and spike shown in the following code:

assign done_axon = (axon_num == 255); always @(negedge clk, negedge reset_n) begin if(~reset_n) axon_num <= 8'd0; else if(initial_axon_num) axon_num <= 8'd0; else if(inc_axon_num) axon_num <= axon_num + 1'b1; else axon_num <= axon_num; end always @(negedge clk, negedge reset_n) begin if(~reset_n) neuron_num <= 8'd0; else if(initial_neuron_num) neuron_num <= 8'd0; else if(inc_neuron_num) neuron_num <= neuron_num + 1'b1; else neuron_num <= neuron_num; end always @(posedge clk, negedge reset_n) begin if(~reset_n) done_neuron <= 0; else if(neuron_num == 255) done_neuron <= 1; else done_neuron <= 0; end wire reg_en; assign reg_en = (neuron_parameter[112 + axon_num] & axon_spikes[axon_num]); neuron_block neuron_block();

### Testbench

Our testbench read inputs from text files, give them to the module and update data from text file to module.

//Đọc file mem reg [367:0] neuron_parameters [0:255]; initial $readmemb(link_to_CSRAM_data, neuron_parameters); reg [1:0] neuron_instructions [0:255]; initial $readmemb(link_to_neuron_instruction, neuron_instructions); //Tự động đưa data từ file mem vào module always @(neuron_num) begin neuron_parameter = neuron_parameters[neuron_num]; end always @(axon_num) begin neuron_instruction = neuron_instructions[axon_num]; end //Tự động update lại data từ file mem sau mỗi lần tính toán xong cho 1 neuron trong 1 tick always @(posedge clk) begin if(update_potential) neuron_parameters[neuron_num][111:103] <= potential_out; else neuron_parameters[neuron_num][111:103] <= neuron_parameters[neuron_num][111:103]; end //Đọc axon_spikes từ file reg [255:0] mem [0:MAX_NUM_TICK - 1]; initial begin if(MAX_NUM_TICK == 1) $readmemb(link_to_axon_spike, mem); else $readmemb(link_to_many_ticks_axon_spike, mem); end integer row = 0; always @(scheduler_set) begin if(scheduler_set) begin axon_spikes = mem[row]; row = row + 1; // $display("%b", axon_spikes); end end

Our testbench assigns tick = 1 after finish 1 tick and stops after all the ticks are processed automatically.

////////AUTO////////////////////////////////////////////////////////// integer num_tick = 0; reg finish; initial finish = 0; always @(tick) begin num_tick = tick ? num_tick + 1 : num_tick; end //tự động gán tick = 1 sau khi tính xong 1 tick///////////////////////// always @(done) begin if(done) begin repeat(3) @(negedge clk); tick = 1; @(negedge clk); tick = 0; end else tick = tick; end //tự động dừng sau khi đạt đủ số tick///////////////////////////// always @(num_tick, MAX_NUM_TICK, done) begin if((num_tick == MAX_NUM_TICK) & done) begin repeat(2) @(negedge clk); finish = 1; #2; $stop; end end

The testbench also writes outputs to a text file and compares to output from software.

///////compare with output from software//////////////////////////// reg [255:0] output_soft [0:MAX_NUM_TICK - 1]; reg wrong; initial wrong = 0; initial $readmemb(link_to_output_soft, output_soft); integer i, j; always @(finish) begin if(finish) begin for(i = 0; i < MAX_NUM_TICK; i = i + 1) begin for(j = 0; j < 256; j = j + 1) begin if(output_hard[i][j] != output_soft[i][j]) begin $display("Error at neuron %d, tick %d", j, i); wrong = 1; end end end end end always @(finish) begin if(finish) begin #1; if(~wrong) $display("Test pass without error"); end end

# RESULT

Table 1 Synthesis results table

| Section | Combination of RANC Token Controller and Neuron Block | Neuron Grid |
| --- | --- | --- |
| Target Chip Technology | Zynq UltraScale+ MPSoC ZCU106 Evaluation | Zynq UltraScale+ MPSoC ZCU106 Evaluation |
| Maximum Frequency | 278.86 MHz | 331.90 MHz |
| Hardware Usage | 235 LUTs, 37 FFs | 277 LUTs, 30 FFs |
| Detecting Throughput | 676.6 Mbps | 805.3 Mbps |
| Latency | 66050 clocks | 66048 clocks |

Table 1 shows the results of our Neuron Grid comparing to Combination of RANC Token Controller and Neuron Block after synthesizing on the same target chip technology Zynq UltraScale+ MPSoC ZCU106 Evaluation.

Our design has higher maximum frequency and detecting throughput (331.90 MHz and 805.3 Mbps comparing to 278.86 MHz and 676.6 Mbps). Our Neuron Grid’s latency also has 2 clocks fewer than RANC’s. Our design uses fewer FFs but more LUTS than RANC’s.

# CONCLUSION

Eventually, the Neuron Grid is successfully designed and meet the desired specification. Although we have to deal with some problems in progress, consequently we have found the way to handle them. For further improvement, we will try to propose a more advanced method to replace our ideal components to achieve a better performance of simulated results. Moreover, throughout this assignment, we have gained a deeper knowledge about those concept. Anyway, nothing is perfect, perhaps our performance is not the most optimized way against this topic. Nevertheless, we hope our project contributes a significant part to the Digital Design, also helps other students to shorten their approach with designing a Neuron Grid. The process of learning and learning how to build a system, a term of simulation that seems easy but very complex and a feat to achieve professionalism. However, under the guidance of Prof. Nguyen Duc Minh, our team has learned the steps to create an economical project with practical application in the most effective way. Along with that, teamwork also helps us to perfect more skills such as arranging time, assigning work to each member, using software for teamwork, ... Our team sincerely thanks for the direction and very practical and straightforward instructions and suggestions from teachers during class presentations, through which we have a more accurate and clear orientation on the way to build products that meet market needs and have practical application. Due to the limited time, the simulation we build is still flawed, we look forward to having your feedback on the product so that we can try harder in the next projects. Once again, my group would like to thank you very much.

Once again, thank you very much for helping us.

REFERENCES

| [1] | "Spiking neural network," [Online]. Available: https://en.wikipedia.org/wiki/Spiking_neural_network. |
| --- | --- |
| [2] | Valancius, et al., "FPGA Based Emulation Environment for Neuromorphic Architectures," 2020 IEEE International Parallel and Distributed Processing Symposium Workshops (IPDPSW), New Orleans, LA, USA, pp. 90-97, 2020. |
| [3] | Valancius, et al., "RANC: Reconfigurable Architecture for Neuromorphic Computing," 2020. |